Fixed-Point FDN Reverb

reverb.c / reverb.h — Integration & Reference Guide

# Overview

This module implements a 4-channel Feedback Delay Network (FDN) reverb in fully fixed-point C. It targets embedded systems running at 44100 Hz with 16-bit signed PCM — specifically the ESP32-C3 and similar single-core RISC-V or ARM Cortex-M parts — but it will compile and run without modification on any C99 platform.

The core algorithm uses a 4×4 Hadamard mixing matrix applied to four delay lines. The Hadamard matrix collapses to a butterfly network (8 integer adds), making it computationally trivial. A one-pole lowpass filter in each feedback path models air absorption. The result sounds substantially better than Schroeder/Freeverb at comparable CPU cost.

# Algorithm

## FDN Topology

A Feedback Delay Network routes N delay lines through an N×N mixing matrix before feeding back. Energy preservation (required for stable, natural-sounding decay) is guaranteed when the matrix is orthogonal. The Hadamard matrix is orthogonal and requires only additions to compute — no multiplies.

Signal flow per sample:

input ──┬──► delay[0] ──► LPF ──► gain ──┐

├──► delay[1] ──► LPF ──► gain ──┤

├──► delay[2] ──► LPF ──► gain ──┤

└──► delay[3] ──► LPF ──► gain ──┘

│

Hadamard ◄─────────────┘

│

wet/dry mix

│

L / R out (decorrelated taps)

## Fixed-Point Format

All coefficients use Q1.14 format: a 16-bit integer where 0x4000 represents 1.0 and the range is approximately [−2, +2). Multiplications are performed in 32-bit and right-shifted by 14.

Internal accumulation uses 32-bit to prevent overflow in the Hadamard butterfly. Output is saturated back to 16-bit with a simple clamp.

## Hadamard Butterfly

The 4×4 Hadamard transform is factored into two stages of butterfly operations. This costs 8 integer adds and zero multiplies, making it essentially free on any platform:

/* Stage 1 */

a = v[0] + v[1];   b = v[0] - v[1];

c = v[2] + v[3];   d = v[2] - v[3];

/* Stage 2 */

v[0] = a + c;  v[1] = b + d;

v[2] = a - c;  v[3] = b - d;

# Source & Downloads

## These Files

The implementation is self-contained in two files:

| File | Contents |
| --- | --- |
| reverb.h | Public API, types, preset enum, reverb_state_t |
| reverb.c | Full implementation — no other sources needed |

## Algorithm References

The following papers describe the algorithms and are freely available online:

| Reference | URL / Notes |
| --- | --- |
| Gardner (1992) — MIT TR #144 "Reverberation Algorithms" | https://www.media.mit.edu/tech-reports/TR-144.pdf |
| Jot & Chaigne (1991) "Digital Delay Networks for Designing Artificial Reverberators" | AES 90th Convention Preprint 3030. Search AES E-Library: aes.org/e-lib |
| Schlecht & Habets (2017) "On Lossless Feedback Delay Networks" | https://doi.org/10.1109/TSP.2017.2699371 |
| Freeverb source (Jezar, 2000) Schroeder/Moorer baseline reference | https://ccrma.stanford.edu/~jos/pasp/Freeverb.html |
| Dattorro (1997) "Effect Design Part 1: Reverberator" | https://ccrma.stanford.edu/~dattorro/EffectDesignPart1.pdf |

Note: If you want to go beyond this FDN implementation, Dattorro is the next step up in quality. The modulated allpass sections require interpolated delay reads in fixed-point, which is the main added complexity over this module.

# Integration

## Drop-In Usage

Copy reverb.c and reverb.h into your project. No other files are needed. Add reverb.c to your build system.

Minimal example — mono or stereo passthrough with reverb:

#include "reverb.h"

reverb_state_t rv;

void app_init(void) {

reverb_init(&rv, REVERB_PRESET_ROOM);

}

/* Call from your audio ISR or DMA callback at 44100 Hz */

void audio_callback(int16_t in_l, int16_t in_r,

int16_t *out_l, int16_t *out_r) {

reverb_process(&rv, in_l, in_r, out_l, out_r);

}

## ESP32-C3 / FreeRTOS Notes

The function reverb_process() is reentrant with respect to its state pointer. Call it from your audio task or I2S DMA callback. Do not share a single reverb_state_t between tasks without a mutex.

Expected CPU load at 44100 Hz, 160 MHz RISC-V (estimated from cycle counts on comparable Cortex-M4):

| Condition | Approx. CPU Load |
| --- | --- |
| reverb_process() alone | ~3–5% at 44100 Hz |
| With VS1053 SPI decode offload | Comfortable — C3 is just the control plane |
| Software MP3 decode + reverb | Tight but feasible; pin reverb to core 0 |

## RAM Budget

All delay memory is statically allocated inside reverb_state_t. No heap allocation.

| Item | Size |
| --- | --- |
| 4 delay lines × 2048 × 2 bytes | 16,384 bytes (16 KB) |
| Pre-delay buffer (256 × 2 bytes) | 512 bytes |
| State variables (gains, pos, etc.) | ~80 bytes |
| Total per instance | ~17 KB |

Tip: Place reverb_state_t in DRAM (not IRAM). The delay line accesses are sequential per sample — cache-friendly on most architectures. If you need to reduce RAM, halve REVERB_MAX_DELAY to 1024; tails will shorten slightly.

# Presets

Four presets are provided. Pass the enum to reverb_init().

| Enum | RT60 (approx) | Default Wet Mix | Character |
| --- | --- | --- | --- |
| REVERB_PRESET_ROOM | ~0.4 s | 50% | Small room, subtle, natural |
| REVERB_PRESET_HALL | ~1.2 s | 62% | Concert hall, open, lush |
| REVERB_PRESET_PLATE | ~0.7 s | 75% | Dense, fast build, vintage plate character |
| REVERB_PRESET_CAVE | ~2 s | 62% | Long, dark, atmospheric |

# Runtime Tuning

## Wet/Dry Mix

/* 50% wet */

reverb_set_mix(&rv, 0x2000);

/* 100% wet (effect only, no dry signal) */

reverb_set_mix(&rv, 0x4000);

/* 25% wet */

reverb_set_mix(&rv, 0x1000);

## Decay / Feedback Gain

reverb_set_gain() sets all four delay lines to the same coefficient. For asymmetric decay (slightly different per line), modify rv.gain[] directly after reverb_init().

/* Shorter decay */

reverb_set_gain(&rv, 0x3000);

/* Longer decay */

reverb_set_gain(&rv, 0x3C00);

/* Do not exceed 0x3FFF — instability */

## Changing Presets at Runtime

reverb_init() zeros all delay lines, which produces a brief silent gap (~delay_length/44100 seconds). For live audio, crossfade two instances if seamless switching is required. For non-critical use, calling reverb_init() mid-stream is fine.

# Porting Notes

| Topic | Notes |
| --- | --- |
| Sample rate | Designed for 44100 Hz. For 48000 Hz, scale all delay[] values by 48000/44100 ≈ 1.088. Pre-delay buffer may need enlarging. |
| Bit depth | Input/output are int16_t. Internal accumulation is int32_t. Feeding 24-bit samples: right-shift input by 8 before calling, left-shift output by 8 after. |
| REVERB_MAX_DELAY | Must be a power of 2. Increase for longer tails (costs 2×REVERB_MAX_DELAY bytes per delay line, ×4 lines). Decrease to save RAM at the cost of shorter max decay. |
| Compiler | Requires C99 or later. No VLAs, no non-standard extensions. Compiles clean with -Wall -Wextra on GCC and Clang. |
| SIMD / DSP intrinsics | The Hadamard and LPF sections are the inner loop. If your target has SIMD (NEON, Helium, XTENSA) the four-lane structure maps directly to 4-wide SIMD ops. |

# Known Limitations

- No modulation on the delay lines. This means no chorus/shimmer effect and the tail can sound slightly static on sustained tones. Adding a small LFO-modulated allpass (Dattorro-style) would fix this at modest extra cost.

- Early reflections are not modelled. The FDN produces diffuse late reverberation only. For a realistic room sound, add a small tapped delay line (6–12 taps, <30 ms) before the FDN input.

- Stereo decorrelation is simple (different output tap pairs). It is adequate but not as wide as a true dual-FDN or Dattorro figure-8 topology.

- RT60 estimates in the preset table are approximate. Actual decay time depends on signal content and the interaction between the LPF and gain coefficients.